Two-Spool Turbofan Performance and HPT Cooling-Bleed Trade Study

Shared engine definition

- Notional two-spool, separate-flow turbofan

- Fan/bypass stream and core stream use separate nozzles

- HPT drives the high-pressure compressor

- LPT drives the fan and low-pressure compressor

- Compressor bleed supplies HPT cooling

- Primary design condition: sea-level takeoff

- Optional second condition: cruise

- No proprietary engine is being replicated

- No full transient control model

- No detailed compressor or turbine maps in the required scope

- No full conjugate CFD in the required scope

Objective

Develop a modular MATLAB model of a two-spool turbofan and quantify how HPT cooling bleed affects engine thrust, specific fuel consumption and turbine operating conditions.

Core inputs

Flight condition

- Altitude

- Flight Mach number

- Ambient temperature and pressure

Engine architecture

- Core mass flow

- Bypass ratio

- Fan pressure ratio

- LPC pressure ratio

- HPC pressure ratio

- Turbine inlet temperature

- Combustor pressure loss

- Component efficiencies

- Mechanical efficiencies

- Fuel heating value

- Cooling-bleed fraction

MATLAB modules/structure

Project_1_turbofan/

- Main_turbofan.m

- Inputs/

  - Baseline_engine.m

- Functions/

  - Atmosphere.m

  - Diffuser.m

  - Compressor.m

  - Combustor.m

  - Cooling_mixer.m

  - Turbine.m

  - Nozzle.m

  - Thrust.m

  - Hpt_stage_sizing.m

  - Validate_balances.m

- Studies/

  - Cooling_bleed_sweep.m

  - Turbine_temperature_sweep.m

  - Pressure_ratio_sweep.m

- Tests/

- Figures/

- Report/

Engine station structure

0	=	 freestream

2	=	fan inlet

13	=	fan/bypass exit

25	=	HPC inlet

3	=	HPC

4	=	combustor exit

41	=	HPT rotor inlet after coolant

mixing

45	=	HPT exit

5	=	LPT exit

8/9	=	core-nozzle throat/exit

18/19	=	bypass-nozzle throat/exit

Required outputs

- Station-by-station total temperature

- Station-by-station total pressure

- Core and bypass mass flows

- Fuel-air ratio

- HPT and LPT work

- Core and bypass nozzle velocities

- Net thrust

- Specific thrust

- TSFC

- Thermal and propulsive efficiency

- Cooling-bleed penalty

- Mass- and energy-balance residuals

Primary trade study

Vary cooling-bleed fraction and plot:

- Net thrust versus cooling bleed

- TSFC versus cooling bleed

- HPT inlet mixed temperature versus cooling bleed

- HPT work and pressure ratio versus cooling bleed

The bleed range will be chosen from cited public literature

## Preliminary HPT sizing bridge

Project 1 also needs a small mean-line sizing module.

From HPT work and chosen nondimensional design parameters:

ψ=Δh0U2

we obtain blade speed:

U=Δh0ψ

Then, using shaft angular speed:

rm=Uω

Mass flow and chosen axial velocity provide an approximate annulus area:

A=mρVx

That lets us estimate:

- Mean blade radius

- Hub and tip radii

- Blade span

- Approximate axial velocity

- Approximate rotor inlet and exit velocity triangles

- Gas-pressure loading basis

Project controls/scope

Required MVP

- Sea-level static/takeoff design point

- Constant-property ideal-gas model

- Two spools and separate exhaust streams

- Prescribed component pressure ratios and efficiencies

- HPT cooling bleed

- Thrust, TSFC, station states, shaft balances

- HPT boundary-condition export

Core finished project

- Cooling-bleed trade study

- Preliminary HPT mean-line sizing

- Validation and balance checks

- Clean report figures

Stretch only

- Cruise operating point

- Temperature-dependent properties

- Component maps

- Optimization

- Transient behavior

Success criteria

Project 1 is considered functional when:

- All station temperatures, pressures, and mass flows are physically valid.

- Mass and shaft-energy residuals are below approximately 0.1%.

- The baseline cycle produces plausible thrust and TSFC trends.

- Cooling bleed causes explainable changes in thrust, TSFC, and HPT conditions.

- Project 1 successfully exports every input needed by Project 2.

- At least one published cycle or textbook case is reproduced closely enough to validate the implementation.

Assumptions log

| Assumption | Value/model | Source | Reason | Sensitivity tested? |
| --- | --- | --- | --- | --- |
| Cold-flow (cp) | Constant | TBD | MVP simplicity | Later |
| Compressor efficiency | TBD | Literature | Notional engine | Yes |
| Combustor pressure loss | TBD | Literature | Preliminary cycle | Yes |
| Cooling fraction | Swept | Literature | Main trade variable | Yes |